ABSTRACT

Gas leakage in classrooms, laboratories, and daycare environments poses serious risks, including fire hazards, explosions, and respiratory problems. Early detection is crucial to ensure the safety of children and staff.

This project presents an IoT-based Gas Leakage Detection and Early Warning System. The system uses:

- MQ-2 Gas Sensor to detect LPG, methane, and smoke.

- ESP32 Microcontroller for processing sensor data.

- Buzzer and red/green LEDs for audible and visual alerts.

- Optional SIM800L GSM module for sending SMS alerts to caregivers or responsible staff.

The device continuously monitors gas concentration levels. If the readings exceed a predefined safety threshold, the red LED lights up, the buzzer activates, and an SMS alert is sent. When conditions are safe, the green LED indicates normal operation.

Testing demonstrated that the system can detect gas leaks early, provide immediate alerts, and significantly reduce response time, thus preventing potential accidents. This project shows that affordable IoT technology can enhance safety in schools, laboratories, and daycare centres.

ACKNOWLEDGEMENT

We sincerely thank the Almighty for seeing us through this project. We extend our gratitude to our integrated science teacher and patron, Tr. Namisi, for guidance throughout this project. We also appreciate our parents, Annet Kingori and Edith Wamae, for encouragement and support. Lastly, we thank the school administration through Mr. Mwaniki for allowing us to participate in KSEF and providing material support. Special thanks to classmates who assisted during testing and data collection.

CHAPTER ONE: INTRODUCTION

1.1 Background Information

Gas leaks are a significant safety concern in indoor environments such as classrooms, daycare centres, and laboratories. Combustible gases like LPG and methane can ignite easily and cause fires or explosions, while some gases pose health risks due to inhalation. Children are particularly vulnerable due to their limited awareness of danger.

Automated gas detection systems using MQ-series sensors offer a practical and affordable solution. By continuously monitoring gas concentrations and providing immediate audible, visual, and remote alerts, such systems help prevent accidents and ensure safety.

1.2 Statement of the Problem

Unnoticed gas leaks in classrooms, laboratories, or daycare centres can result in fires, explosions, and health hazards. Human observation alone is insufficient for early detection, especially in busy environments.

1.3 Objectives

- To detect the presence of combustible gases such as LPG and methane using an MQ-2 sensor.

- To provide instant visual (LED) and audible (buzzer) alerts when unsafe gas levels are detected.

- To send SMS alerts to responsible staff or caregivers in the event of dangerous gas concentrations.

- To demonstrate a low-cost, IoT-based safety system suitable for schools and daycare centres.

1.4 Hypothesis

- If the MQ gas sensor detects gas concentrations above a defined threshold, the system will activate a buzzer and red LED within seconds.

- Green LED will indicate safe conditions.

- SMS alerts will be delivered rapidly via the GSM module to ensure immediate response by responsible personnel.

1.5 Significance of the Project

- Prevents accidents and potential injuries due to gas leaks.

- Provides affordable, low-cost safety monitoring suitable for schools and daycare centres.

- Demonstrates practical application of IoT and embedded systems for safety.

- Supports SDG 3: Good Health and Well-being by improving indoor safety.

- Promotes innovation and digital literacy among young Kenyan students.

1.6 Limitations

- Sensors require proper calibration for accurate readings.

- GSM or Wi-Fi network required for SMS alerts.

- Cannot detect gases outside the MQ-2 sensor range.

1.7 Assumptions

- Sensors are correctly installed and calibrated.

- The device is placed in a location where gas leaks can be detected early.

- Staff respond promptly to alerts.

1.8 Precautions

- Avoid placing the setup near strong air drafts that may affect readings.

- Handle the sensor and electronics carefully to prevent damage.

CHAPTER TWO: LITERATURE REVIEW

2.1 Introduction

Gas leaks are a common safety concern. Studies show that LPG and methane are flammable and pose significant risks in indoor environments such as classrooms, laboratories, and daycare centers. Early detection is critical to prevent fire hazards, explosions, and health issues.

2.2 Gas Detection Technologies

MQ-series sensors (MQ-2, MQ-5, MQ-135) are widely used for detecting combustible gases. They operate by detecting changes in resistance when exposed to gas. MQ-2 is suitable for detecting LPG, methane, propane, and smoke. Research shows that these sensors are:

- Reliable for indoor gas detection

- Affordable and easily integrated with microcontrollers

- Capable of triggering alarms when gas exceeds safety thresholds

2.3 Microcontroller-Based Monitoring Systems

Microcontrollers like ESP32 and Arduino provide real-time processing for gas detection systems. They enable:

- Continuous monitoring

- Comparison of sensor readings with safety thresholds

- Activation of visual (LED) and audible (buzzer) alerts

- Optional SMS notifications via GSM modules

IoT-enabled microcontroller systems improve response time and reliability.

2.4 Research Gap

Industrial gas detection systems are expensive and not suitable for schools or daycare centres. There is a need for a low-cost, affordable, and reliable gas detection system for indoor child safety. This project addresses that gap.

CHAPTER THREE: METHODOLOGY

3.1 Apparatus

- ESP32 microcontroller

- MQ-2 gas sensor

- Buzzer

- Red LED (danger indicator)

- Green LED (safe indicator)

- Resistors (220Ω)

- Jumper wires, breadboard

- Power supply

3.2 Procedure

- Connect the MQ-2 sensor analog output to the ESP32 analog pin.

- Connect the buzzer to a digital pin.

- Connect the red and green LEDs with 220Ω resistors to digital pins.

- Program ESP32 to continuously read the sensor.

- Set a threshold for safe gas levels.

- If gas exceeds the threshold:

  - Activate red LED

  - Turn off green LED

  - Activate buzzer

  - Send SMS alert (optional)

- Test the system in a controlled environment with safe gas exposure.

3.3 Detection Logic

- Safe Conditions: Green LED ON, Red LED OFF, Buzzer OFF

- Danger Conditions: Red LED ON, Green LED OFF, Buzzer ON, SMS alert sent

3.4 Variables

Independent Variables:

- Gas concentration measured by MQ-2 sensor

Dependent Variables:

- Activation of buzzer

- Red/green LED status

- SMS alert delivery

CHAPTER FOUR: DATA ANALYSIS AND INTERPRETATION

Table 1: Gas Level vs Alert Trigger

| Gas Level (ppm) | Status | LED | Buzzer | SMS Alert |
| --- | --- | --- | --- | --- |
| 0–200 | Safe | Green ON | OFF | No |
| 201–500 | Warning | Red ON | ON | Yes |
| >500 | Danger | Red ON | ON | Yes |

Analysis:

- System accurately detects gas levels exceeding threshold.

- Buzzer and red LED provide immediate warning.

- SMS alerts confirm rapid notification of responsible staff.

Discussion:
The system successfully monitors gas concentrations and provides early warning. Calibration ensures sensitivity while minimizing false alarms. The combination of visual, audible, and remote alerts provides comprehensive safety.

CHAPTER FIVE: CONCLUSION AND RECOMMENDATIONS

5.1 Conclusion

The Gas Leakage Detection and Early Warning System was successfully designed, tested, and implemented. It provides early alerts through LED, buzzer, and SMS notifications when gas levels exceed safe thresholds. This improves safety in classrooms, laboratories, and daycare centres by preventing accidents and health risks.

5.2 Recommendations

- Schools, daycare centres, and laboratories should adopt low-cost gas detection systems to improve indoor safety.

- The Kenya National Bureau of Standards (KEBS) should develop guidelines for affordable safety monitoring systems in educational settings.

- Integration with GSM or Wi-Fi modules can enhance alert delivery in areas with weak mobile signals.

- Future enhancements could include multiple sensors, data logging, and IoT dashboards for real-time monitoring.

REFERENCES

Alasmari, N., et al. (2025). Using Internet of Things for child and school safety: A systematic review. International Journal of Occupational and Environmental Medicine, 16(1), 3–12.

Herlea, D. M., Iancu, B., & Ardelean, E. R. (2025). Low-cost gas detection using MQ sensors and microcontrollers. Informatics, 12(2), 50.

Irvin, D. W., Luo, Y., Huffman, J. M., Grasley-Boy, N., Rous, B., & Hansen, J. H. L. (2021). Capturing environmental hazards in classrooms: Advances in measuring features of safety systems. Early Childhood Research Quarterly, 57, 102–109.

APPENDIX: CIRCUIT DIAGRAM AND SENSOR LAYOUT

Components Used:

- MQ-2 sensor: detects LPG, methane, smoke

- ESP32: processes readings

- Buzzer: audible alarm

- Red LED: danger indicator

- Green LED: safe indicator